# From Privileged RL to VLA Data at Scale: Sim Rollout Distillation for LeKiwi

Workshop Poster Draft (v0.2) — 2026-02-13

Authors: <Your Name(s)>  |  Affiliation: <Lab/University>

# One-sentence takeaway

A privileged-observation RL teacher in simulation generates scalable, high-success rollouts that improve real-world VLA fine-tuning with minimal teleoperation.

# Abstract (120–150 words)

Vision–Language–Action (VLA) policies require large, high-quality (image, action) datasets, which are typically collected via human teleoperation and do not scale. We propose a practical systems pipeline that uses simulation as a data factory: a privileged-observation RL policy is trained in Isaac Lab for a mobile manipulator (LeKiwi), then rolled out at high success rate under domain randomization to produce large-scale, automatically labeled trajectories. These rollouts are distilled into VLA students (π0Fast and Isaac-GR00T) via supervised fine-tuning on (image, instruction, proprioceptive state) → action data, and transferred to the real robot. We study how rollout quantity (1K vs 10K episodes) and domain randomization strength (weak vs strong) interact for sim-to-real performance, and compare teacher-generated rollouts to baseline data sources such as teleop-only SFT and behavior-cloning rollouts.

# Motivation

• Teleoperation data collection is expensive and hard to scale.
• Pure behavior cloning (BC) suffers from compounding error when used as a rollout generator.
• Simulation can generate large data volumes and controlled diversity (domain randomization), but we need a reliable policy to produce high-success trajectories.

# Method: Teacher → Rollouts → Student

1) Build Real2Sim environment in Isaac Sim/Isaac Lab (robot geometry, cameras, sensor setup).
2) Calibrate simulation dynamics against the real robot (wheel/arm parameter measurement, dynamics tuning, replay verification).
3) Collect a small set of teleop demonstrations in the calibrated sim.
4) Train a BC policy and use it to warm-start RL.
5) Train a privileged-observation RL teacher (PPO) with manual shaped reward (phase FSM + shaping + GRASP timeout for retry).
6) Roll out the teacher at scale under domain randomization; record images, instructions, and actions.
7) Fine-tune VLA students (π0Fast, GR00T) on the generated dataset.
8) Evaluate sim and real success rates; analyze failure modes and the effect of quantity vs diversity.

# Claims (what we do / do not claim)

We claim:

• Privileged RL can act as a high-quality rollout generator for VLA fine-tuning in our setting.

• We provide empirical evidence on how rollout quantity and DR strength interact for sim-to-real transfer.

• We compare π0Fast vs GR00T students under matched data conditions.

We do NOT claim:

• Automatic reward learning (IRL) or reward-engineering-free training.

• Multi-robot generality or broad multi-task generalist VLA.

# Experimental Setup

Task: Fetch (SEARCH → APPROACH → GRASP → RETURN) with navigation + manipulation.

Sim calibration: Real robot dynamics (wheel radius, base radius, arm joint limits/friction) are measured and used to tune Isaac Sim physics parameters before any policy training. All RL training and rollout collection use calibrated dynamics.

Sim-real validated constants in this run: WHEEL_RADIUS=0.049m, BASE_RADIUS=0.1085m, LIN_SCALE=1.0166, ANG_SCALE=1.2360, WZ_SIGN=-1.0.

Supplementary check: sim_real_calibration_test.py validates pose-level distance/rotation only; deployment Go/NoGo is finalized by tune_sim_dynamics.py + replay_in_sim.py + check_calibration_gate.py.

Synchronization: with AUTO_LOAD_COMP_FROM_DYNAMICS=True, sim_real_calibration_test.py first loads command_transform from calibration/tuned_dynamics.json and falls back to hardcoded constants if missing.

Usage rule: rollout data collection keeps sim-space actions; real deployment applies one sim->real inverse transform to base action [vx, vy, wz] right before robot command output.

Current inverse (M=identity): vx_real=vx_sim/1.0166, vy_real=vy_sim/1.0166, wz_real=wz_sim/(-1.2360).

Teacher RL: Isaac Lab DirectRLEnv, privileged state observation (33D/37D), action (9D). Physics-based grasp (contact sensor + gripper close + adaptive distance), GRASP timeout for retry.

Student VLA inputs: camera images (base + wrist) + instruction + robot_state (9D: arm joint positions 6 + wheel velocities 3); outputs: robot actions (9D: base vx/vy/wz + arm 6 joint targets).

Domain Randomization:

• Weak DR: limited variation in lighting intensity/color, object pose/scale.
• Strong DR: wider lighting/color-temperature ranges + object diversity (1030 USD objects) + distractor objects. Future: texture randomization, camera noise/jitter.

# Planned Conditions (minimal poster matrix)

| ID | Data source | DR | Episodes | Student | Purpose |
| --- | --- | --- | --- | --- | --- |
| C1 | Human teleop | weak | 50 | π0Fast | SFT baseline (low) |
| C2 | Human teleop | weak | 200 | π0Fast | SFT baseline (high) |
| C3 | BC rollouts | weak | 1K | π0Fast | BC as generator |
| C4 | RL teacher rollouts | weak | 1K | π0Fast | Teacher vs BC |
| C5 | RL teacher rollouts | weak | 10K | π0Fast | Quantity scaling |
| C6 | RL teacher rollouts | strong | 1K | π0Fast | Diversity effect |
| C7 | RL teacher rollouts | strong | 10K | π0Fast | Quantity×diversity |
| C8 | RL teacher rollouts | weak | 10K | GR00T | Student comparison |

# Metrics and Reporting

Primary: Real success rate (n ≥ 30 trials; report 95% binomial confidence intervals).

Secondary: Sim success rate, rollout generation cost, failure taxonomy, and qualitative sim-vs-real visuals.

# Human-effort ledger (report transparently)

| Item | Unit | Value | Notes |
| --- | --- | --- | --- |
| Sim build & calibration | hours |  | URDF/USD, camera, physics tuning |
| Reward shaping & tuning | hours |  | FSM + shaping parameters |
| Teleop demos | minutes |  | 50/200 episodes |
| Teacher RL training | GPU-hours |  | num_envs=2048 headless |
| Rollout generation | GPU-hours |  | 1K/10K episodes |
| Student fine-tuning | GPU-hours |  | π0Fast / GR00T |

# Limitations and Future Work

• Manual reward shaping is used; automating reward learning (IRL/LLM reward) is future work.

• Single robot and a single complex task; extending to more tasks/robots is future work.

• Transfer depends on sim fidelity and DR choices; we analyze DR ablations but do not solve sim-to-real in general.

# Poster Layout (suggested)

Top: Title + takeaway
Left: Motivation + Related work
Center: Pipeline diagram + Teacher RL setup
Right: Main table (C1–C8) + Quantity×DR plot
Bottom: Human-effort ledger + Failure analysis + Limitations

# References (placeholders)

• RLDG (RL-generated data for fine-tuning generalist policies)
• MimicGen (demo augmentation in sim)
• π0Fast / Isaac-GR00T docs